# Models File

# University of Florida SWAMP Center

# Last Updated 11/16/2010, Daniel Cummings

#=======================================================

# Section 1. Contact Models

#=======================================================

#--------------------------------

# 1.1 Ohmic Contacts

#--------------------------------

# standard ohmic contact

proc ohmic.contact {Contact} {

set vt 0.02558357

set ni 1.1e10

pdbSetBoolean $Contact Elec Flux 1

pdbSetBoolean $Contact Hole Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetBoolean $Contact Elec Fixed 1

pdbSetBoolean $Contact Hole Fixed 1

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetDouble $Contact Elec Flux.Scale 1.619e-19

pdbSetDouble $Contact Hole Flux.Scale 1.619e-19

pdbSetString $Contact DevPsi Equation "ND - NA - Elec + Hole"

pdbSetString $Contact Elec Equation "DevPsi - $vt*log((Elec)/$ni) -$Contact"

pdbSetString $Contact Hole Equation "DevPsi + $vt*log((Hole)/$ni) -$Contact"

}

#--------------------------------

# Quasi-Fermi Contacts

#--------------------------------

proc qf.ohmic.contact {Contact} {

set vt 0.02558357

set ni 1.1e10

pdbSetBoolean $Contact Qfn Flux 1

pdbSetBoolean $Contact Qfp Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetBoolean $Contact Qfn Fixed 1

pdbSetBoolean $Contact Qfp Fixed 1

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetDouble $Contact Qfp Flux.Scale 1.619e-19

pdbSetDouble $Contact Qfn Flux.Scale 1.619e-19

pdbSetString $Contact DevPsi Equation "ND - NA - $ni*exp((DevPsi-Qfn)/$vt) + $ni*exp((Qfp-DevPsi)/$vt)"

pdbSetString $Contact Qfn Equation "Qfn-$Contact"

pdbSetString $Contact Qfp Equation "Qfp-$Contact"

pdbSetString $Contact Equation "1.619e-19 * (Flux_Qfp - Flux_Qfn)"

}

#--------------------------------

# Schottky Contacts - Band-Based

#--------------------------------

pdbSetDouble Silicon Egcond 0.55

pdbSetDouble Silicon Egval -0.55

pdbSetDouble Silicon Ncond 2.51e19

pdbSetDouble Silicon Nval 2.51e19

term add name=Econd eqn = "[pdbDelayDouble Si Egcond] - DevPsi" Si

term add name=Eval eqn = "[pdbDelayDouble Si Egval] - DevPsi" Si

proc n.sch.contact {Mat Contact Rvel} {

#ohmic contacts fix all quantities

pdbSetBoolean $Contact Elec Fixed 0

pdbSetBoolean $Contact Hole Fixed 0

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetBoolean $Contact Elec Flux 1

pdbSetBoolean $Contact Hole Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetDouble $Contact Hole Flux.Scale 1.619e-19

pdbSetDouble $Contact Elec Flux.Scale 1.619e-19

#charge neutrality for the dominant carrier w/ Recombination velocity

pdbSetString $Contact Elec Equation "$Rvel*(Elec-ND)"

#set quasi-fermi level for electrons to the applied bias at the contact

set Econd "[pdbDelayDouble $Mat Egcond] - DevPsi"

set Nc "[pdbDelayDouble $Mat Ncond]"

set eqn "$Econd + 0.025 * log( (ND+1.0e-15) / $Nc) + $Contact"

pdbSetString $Contact DevPsi Equation $eqn

#set quasi-fermi level for holes to the applied bias at the contact

pdbSetString $Contact Hole Equation "$Rvel*(Hole - 1.0e20/ND)"

#external current

pdbSetString $Contact Equation "1.619e-19 * (Flux_Hole - Flux_Elec)"

}

proc p.sch.contact {Mat Contact Rvel} {

#ohmic contacts fix all quantities

pdbSetBoolean $Contact Elec Fixed 0

pdbSetBoolean $Contact Hole Fixed 0

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetBoolean $Contact Elec Flux 1

pdbSetBoolean $Contact Hole Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetDouble $Contact Hole Flux.Scale 1.619e-19

pdbSetDouble $Contact Elec Flux.Scale 1.619e-19

#charge neutrality for the dominant carrier w/ Recombination velocity

pdbSetString $Contact Hole Equation "$Rvel*(Hole-NA)"

#set quasi-fermi level for electrons to the applied bias at the contact

set Eval "-[pdbDelayDouble $Mat Egval] + DevPsi"

set Nv "[pdbDelayDouble $Mat Nval]"

set eqn "$Eval + 0.025 * log( (NA+1.0e-15) / $Nv) + $Contact"

pdbSetString $Contact DevPsi Equation $eqn

#set quasi-fermi level for holes to the applied bias at the contact

pdbSetString $Contact Elec Equation "$Rvel*(Elec - 1.0e20/NA)"

#external current

pdbSetString $Contact Equation "1.619e-19 * (Flux_Hole - Flux_Elec)"

}

proc p.sch {Mat Contact Rvel} {

#ohmic contacts fix all quantities

pdbSetBoolean $Contact Elec Fixed 0

pdbSetBoolean $Contact Hole Fixed 0

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetBoolean $Contact Elec Flux 1

pdbSetBoolean $Contact Hole Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetDouble $Contact Hole Flux.Scale 1.619e-19

pdbSetDouble $Contact Elec Flux.Scale 1.619e-19

#charge neutrality for the dominant carrier w/ Recombination velocity

pdbSetString $Contact Hole Equation "$Rvel*(Hole-Hole_DC)"

#set quasi-fermi level for electrons to the applied bias at the contact

set Eval "-[pdbDelayDouble $Mat Egval] + DevPsi"

set Nv "[pdbDelayDouble $Mat Nval]"

set eqn "$Eval + 0.025 * log( (NA+1.0e-15) / $Nv) + $Contact"

pdbSetString $Contact DevPsi Equation $eqn

#set quasi-fermi level for holes to the applied bias at the contact

pdbSetString $Contact Elec Equation "$Rvel*(Elec - Elec_DC)"

#external current

pdbSetString $Contact Equation "1.619e-19 * (Flux_Hole - Flux_Elec)"

}

pdbSetDouble Silicon Egcond 0.55

pdbSetDouble Silicon Egval -0.55

pdbSetDouble Silicon Ncond 2.51e19

pdbSetDouble Silicon Nval 2.51e19

proc n.qf.sch.contact {Mat Contact Rvel} {

set vt 0.02558357

set ni 1.1e10

#ohmic contacts fix all quantities

pdbSetBoolean $Contact Qfn Flux 1

pdbSetBoolean $Contact Qfp Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetBoolean $Contact Qfn Fixed 0

pdbSetBoolean $Contact Qfp Fixed 0

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetDouble $Contact Qfp Flux.Scale 1.619e-19

pdbSetDouble $Contact Qfn Flux.Scale 1.619e-19

#charge neutrality for the dominant carrier w/ Recombination velocity

pdbSetString $Contact Qfn Equation "$Rvel*(($ni*exp((DevPsi-Qfn)/$vt))-ND)"

#set quasi-fermi level for electrons to the applied bias at the contact

set Econd "[pdbDelayDouble $Mat Egcond] - DevPsi"

set Nc "[pdbDelayDouble $Mat Ncond]"

set eqn "$Econd + 0.025 * log( (ND+1.0e-15) / $Nc) + $Contact"

pdbSetString $Contact DevPsi Equation $eqn

pdbSetString $Contact Qfp Equation "$Rvel*(($ni*exp((Qfp-DevPsi)/$vt)) - 1.0e20/ND)"

}

proc p.qf.sch.contact {Mat Contact Rvel} {

set vt 0.02558357

set ni 1.1e10

#ohmic contacts fix all quantities

pdbSetBoolean $Contact Qfn Flux 1

pdbSetBoolean $Contact Qfp Flux 1

pdbSetBoolean $Contact DevPsi Flux 1

pdbSetBoolean $Contact Qfn Fixed 0

pdbSetBoolean $Contact Qfp Fixed 0

pdbSetBoolean $Contact DevPsi Fixed 1

pdbSetDouble $Contact Qfp Flux.Scale 1.619e-19

pdbSetDouble $Contact Qfn Flux.Scale 1.619e-19

#charge neutrality for the dominant carrier w/ Recombination velocity

pdbSetString $Contact Qfp Equation "$Rvel*(($ni*exp((Qfp-DevPsi)/$vt))-NA)"

#set quasi-fermi level for electrons to the applied bias at the contact

set Eval "-[pdbDelayDouble $Mat Egval] + DevPsi"

set Nv "[pdbDelayDouble $Mat Nval]"

set eqn "$Eval + 0.025 * log( (NA+1.0e-15) / $Nv) + $Contact"

pdbSetString $Contact DevPsi Equation $eqn

#set quasi-fermi level for holes to the applied bias at the contact

pdbSetString $Contact Qfn Equation "$Rvel*(($ni*exp((DevPsi-Qfn)/$vt))-(1.0e20/NA))"

}

#=======================================================

# Section 2. Mobility Models

#=======================================================

# 2010 IEEE TNS - Mobility Modelling Considerations for Radiation Effects

proc mobility.radiation {Temp} {

set T $Temp

set T0 300

set	D 1.0400e21

set	F 7.4520e13

term add name=mob_conwell eqn = "($D*(($T/$T0)^(3/2))/sqrt(Elec*Hole))*(log(1+($F*($T/$T0)^2)*(Elec*Hole)^(-1/3)))^(-1)"

term add name=Ntot eqn = "ND+NA"

term add name=Ntote eqn = "ND+NA"

term add name=Ntoth eqn = "ND+NA"

#-----------------------

# Majority Electron

#-----------------------

term add name=mu_max_e eqn = "1417.0"

term add name=mu_e_min1 eqn = "52.2"

term add name=mu_e1 eqn = "39.4"

term add name=Pc_e eqn = "0.0"

term add name=Cr_e eqn = "9.68e16*($T/$T0)^(2.4)"

term add name=a_e eqn = "0.68*($T/$T0)^(-0.4)"

term add name=Cs_e eqn = "3.43e20"

term add name=b_e eqn = "2.0"

term add name=mob1e eqn = "mu_e_min1*($T/$T0)^(-0.57)"

term add name=mob2e eqn = "((mu_max_e-mu_e_min1)*($T/$T0)^(-2.33))/(1+(((Ntote)/Cr_e)^(a_e)))"

term add name=mob3e eqn = "mu_e1/(1+((Cs_e/(Ntote))^(b_e)))"

term add name=Emob_Masetti eqn = "mob1e+mob2e-mob3e"

#-----------------------

# Minority Electron

#-----------------------

#Term 1

term add name=m_mn_eb eqn = "52.2"

term add name=mob1e_min eqn = "m_mn_eb*($T/$T0)^(-0.57)"

#Term 2

term add name=m_mx_eb eqn = "1270.0"

term add name=Cr_e_min eqn = "4.68e16*($T/$T0)^(2.4)"

term add name=a_1_eb eqn = "0.68*($T/$T0)^(-0.146)"

term add name=mob2e_min eqn = "((m_mx_eb-m_mn_eb)*($T/$T0)^(-2.33))/(1+(((Ntoth)/Cr_e_min)^(a_1_eb)))"

#Term 3

set mu_max_eb_min 150.0

set Cr_eb_min 2e20

set a_eb_min 3.7

term add name=mob2eb_min eqn = "($mu_max_eb_min)/(1+(((Ntoth)/$Cr_eb_min)^($a_eb_min)))"

#Term 4

set mu_e1_min 39.0

set Cs_e_min 3.34e20

set b_e_min 2.0

term add name=mob3e_min eqn = "$mu_e1_min/(1+(($Cs_e_min/(Ntoth))^($b_e_min)))"

term add name=Emob_Masetti_min eqn = "mob1e_min+mob2e_min-mob3e_min+mob2eb_min"

term add name=w_ND eqn = "ND/(Ntot)"

term add name=w_NA eqn = "NA/(Ntot)"

term add name=Elec_Sum eqn = "((w_ND/Emob_Masetti)+(w_NA/Emob_Masetti_min))^(-1)"

term add name=Emob_Dan eqn = "(mob_conwell*Elec_Sum)/(mob_conwell+Elec_Sum)"

# ---------------------------

# Hole Majority

# ---------------------------

set m_mx_h 470.5

set m_mn1_h 44.9

set m_mn2_h 44.9

set m_1_h 29.0

set Pc_h 9.23e16

term add name=Cr_h eqn = "2.23e17*($T/$T0)^(2.4)"

set Cs_h 6.1e20

term add name=a_h eqn = "0.719*($T/$T0)^(-0.4)"

set b_h 2.0

term add name=mob1h eqn = "$m_mn1_h*($T/$T0)^(-0.57)"

term add name=mob2h eqn = "(($m_mx_h-$m_mn2_h)*($T/$T0)^(-2.33))/(1+(((Ntoth)/Cr_h)^(a_h)))"

term add name=mob3h eqn = "$m_1_h/(1+(($Cs_h/(Ntoth))^($b_h)))"

term add name=Hmob_Masetti eqn = "mob1h+mob2h-mob3h"

# ---------------------------

# Hole Minority

# ---------------------------

set m_mn_hb 44.9

term add name=mob1hb eqn = "$m_mn_hb*($T/$T0)^(-0.57)"

term add name=a_1_hb eqn = "0.719*($T/$T0)^(-0.4)"

set m_mx_hb 370.0

term add name=Cr_h_min eqn = "1.0e17*($T/$T0)^(2.4)"

term add name=mob2hb eqn = "(($m_mx_hb-$m_mn_hb)*($T/$T0)^(-2.8))/(1+(((Ntoth)/Cr_h_min)^(a_1_hb)))"

set mu_max_hb_min 100.0

set Cr_hb_min 2e20

set a_hb_min 3.7

term add name=mob4hb eqn = "($mu_max_hb_min)/(1+(((Ntoth)/$Cr_hb_min)^($a_hb_min)))"

set mu_h1_min 33.0

set Cs_h_min 3.34e20

set b_h_min 2.0

term add name=mob3hb eqn = "$mu_h1_min/(1+(($Cs_h_min/(Ntoth))^($b_h_min)))"

term add name=Hmob_Masetti_min eqn = "mob1hb+mob2hb+mob4hb-mob3hb"

term add name=Hole_Sum eqn = "((w_NA/Hmob_Masetti)+(w_ND/Hmob_Masetti_min))^(-1)"

term add name=Hmob_Dan eqn = "(mob_conwell*Hole_Sum)/(mob_conwell+Hole_Sum)"

}

# Arora Model

proc mobility.arora {Temp} {

set Amin_e 88

set Amin_h 54.4

set alpham_e -0.57

set alpham_h -0.57

set Ad_e 1252

set Ad_h 407

set alphad_e -2.33

set alphad_h -2.33

set An_e 1.25e17

set An_h 2.35e17

set alphaN_e 2.4

set alphaN_h 2.4

set Aa_e 0.88

set Aa_h 0.88

set alphaa_e -0.146

set alphaa_h -0.146

set T $Temp

set T0 300.0

term add name=mu_min_e eqn = "$Amin_e*($T/$T0)^$alpham_e"

term add name=mu_d_e eqn = "$Ad_e*($T/$T0)^$alphad_e"

term add name=N0_e eqn = "$An_e*($T/$T0)^$alphaN_e"

term add name=Astar_e eqn = "$Aa_e*($T/$T0)^$alphaa_e"

term add name=Emob_Arora eqn = "mu_min_e+(mu_d_e/(1+((ND+NA)/N0_e)^Astar_e))"

term add name=mu_min_h eqn = "$Amin_h*($T/$T0)^$alpham_h"

term add name=mu_d_h eqn = "$Ad_h*($T/$T0)^$alphad_h"

term add name=N0_h eqn = "$An_h*($T/$T0)^$alphaN_h"

term add name=Astar_h eqn = "$Aa_h*($T/$T0)^$alphaa_h"

term add name=Hmob_Arora eqn = "mu_min_h+(mu_d_h/(1+((ND+NA)/N0_h)^Astar_h))"

}

#-------------------------------------------------------

# 2.1 Philips Mobility

#

# D. B. M. KLAASSEN, A UNIFIED MOBILITY MODEL FOR DEVICE SIMULATION

# Solid-State Electronics Vol. 35, No. 7, pp. 953 959, 1992

#

# INPUTS Required:

# 1. ND - Donors

# 2. NA - Acceptors

# 3. Elec - Electrons

# 4. Hole - Holes

# OUTPUT Terms:

# 1. Emob_Philips - Electron Mobility

# 2. Hmob_Philips - Hole Mobility

#

# Notes:

# ND and NA must be at least 1.0 for all regions

#-------------------------------------------------------

proc mobility.philips {temp} {

#------------------------

# 1. Lattice Scattering

#------------------------

set T_0 300.0

set T_Lattice $temp

#Phosphorus

set mu_e_max 1414.0

set mu_min_e 68.5

set theta_e 2.285

set N_eref 9.2e16

set alpha_e 0.711

#Boron

set mu_h_max 470.5

set mu_min_h 44.9

set theta_h 2.247

set N_href 2.23e17

set alpha_h 0.719

term add name=mu_eL eqn = "$mu_e_max*(($T_Lattice/$T_0)^(-$theta_e))"

term add name=mu_hL eqn = "$mu_h_max*(($T_Lattice/$T_0)^(-$theta_h))"

term add name=mu_eN eqn = "(($mu_e_max^2)/($mu_e_max-$mu_min_e))*(($T_Lattice/$T_0)^(3*$alpha_e-1.5))"

term add name=mu_hN eqn = "(($mu_h_max^2)/($mu_h_max-$mu_min_h))*(($T_Lattice/$T_0)^(3*$alpha_h-1.5))"

term add name=mu_ec eqn = "(($mu_e_max*$mu_min_e)/($mu_e_max-$mu_min_e))*(($T_Lattice/$T_0)^0.5)"

term add name=mu_hc eqn = "(($mu_h_max*$mu_min_h)/($mu_h_max-$mu_min_h))*(($T_Lattice/$T_0)^0.5)"

#-------------------------------

# High-Concentration Clustering

#-------------------------------

set c_D 0.21

set c_A 0.5

set N_refD 7.2e20

set N_refA 4.0e20

term add name=ND_clus eqn = "ND*(1+(1/($c_D+(($N_refD/ND)^2))))"

term add name=NA_clus eqn = "NA*(1+(1/($c_A+(($N_refA/NA)^2))))"

term add name=N_esc eqn = "ND_clus + NA_clus + Hole"

term add name=N_hsc eqn = "ND_clus + NA_clus + Elec"

#---------------------

# Screening Parameter

#---------------------

set me_div_m0 1.0

set mh_div_m0 1.258

set f_CW 2.459

set f_BH 3.828

term add name=Pe eqn = "(($f_CW/(3.97*10^13*(N_esc^(-2/3)))+$f_BH/(((1.36*10^20)/(Elec+Hole))*$me_div_m0))^-1)*(($T_Lattice/$T_0)^2)"

term add name=Ph eqn = "(($f_CW/(3.97*10^13*(N_hsc^(-2/3)))+$f_BH/(((1.36*10^20)/(Elec+Hole))*$mh_div_m0))^-1)*(($T_Lattice/$T_0)^2)"

#------------------------------

# Minority Impurity Scattering

#------------------------------

set a_g 0.89233

set b_g 0.41372

set c_g 0.005978

set alpha_g 0.28227

set alphaprime_g 0.72169

set beta_g 0.19778

set gamma_g 1.80618

set m0_div_me 1.0

set m0_div_mh 0.794913

term add name=G_Pe eqn = "1-$a_g/(($b_g+Pe*(($m0_div_me*($T_Lattice/$T_0))^$alpha_g))^$beta_g)+$c_g/((Pe*(($m0_div_me*($T_Lattice/$T_0))^$alphaprime_g))^$gamma_g)"

term add name=G_Ph eqn = "1-$a_g/(($b_g+Ph*(($m0_div_mh*($T_Lattice/$T_0))^$alpha_g))^$beta_g)+$c_g/((Ph*(($m0_div_mh*($T_Lattice/$T_0))^$alphaprime_g))^$gamma_g)"

#--------------------------

# Electron-Hole Scattering

#--------------------------

set me_div_mh 0.794913

set mh_div_me 1.258

term add name=F_Pe eqn = "(0.7643*(Pe^0.6478)+2.2999+6.5502*$me_div_mh)/(Pe^0.6478+2.367-0.8552*$me_div_mh)"

term add name=F_Ph eqn = "(0.7643*(Ph^0.6478)+2.2999+6.5502*$mh_div_me)/(Ph^0.6478+2.367-0.8552*$mh_div_me)"

#--------------------------

# Mobility Model Equations

#--------------------------

term add name=N_e_sceff eqn = "ND_clus+G_Pe*NA_clus+Hole/F_Pe"

term add name=N_h_sceff eqn = "NA_clus+G_Ph*ND_clus+Elec/F_Ph"

term add name=mu_e_DAeh eqn = "mu_eN*(N_esc/N_e_sceff)*(($N_eref/N_esc)^$alpha_e)+mu_ec*((Elec+Hole)/N_e_sceff)"

term add name=mu_h_DAeh eqn = "mu_hN*(N_hsc/N_h_sceff)*(($N_href/N_hsc)^$alpha_h)+mu_hc*((Elec+Hole)/N_h_sceff)"

term add name=Emob_Philips eqn = "(mu_e_DAeh*mu_eL)/(mu_e_DAeh+mu_eL)"

term add name=Hmob_Philips eqn = "(mu_h_DAeh*mu_hL)/(mu_h_DAeh+mu_hL)"

}

#-------------------------------------------------------

# 2.2 Lombardi Mobility

#

#C. Lombardi et al., “A Physically Based Mobility Model

#for Numerical Simulation of Nonplanar Devices."

#IEEE Transactions on Computer-Aided Design, vol. 7, no. 11, pp. 1164–1171, 1988.

#

# INPUTS Required:

# 1. ND - Donors

# 2. NA - Acceptors

# 3. DevPsi - Electrostatic Potential

# OUTPUT Terms:

# 1. Emob_Lombardi - Electron Mobility

# 2. Hmob_Lombardi - Hole Mobility

#-------------------------------------------------------

proc mobility.lombardi {} {

# (using default coefficents for silicon)

set T_lattice 300.0

set T0 300.0

set B_e 4.75e7

set B_h 9.925e6

set C_e 5.8e2

set C_h 2.947e3

set N0 1.0

set lambda_e 0.1250

set lambda_h 0.0317

set k_e 1.0

set k_h 1.0

set delta_e 5.82e14

set delta_h 2.0546e14

set eta_e 5.82e30

set eta_h 2.0546e30

term add name=F_trans eqn = "(abs(dot(DevPsi,x*1.0e-4)) + 1.0e-10)"

#term add name=F_trans eqn = "1.0"

# surface contribution due to acoustic phonon scattering

term add name=mu_phonon_e eqn = "($B_e/F_trans)+($C_e*(((ND+NA)/$N0)^$lambda_e))/((F_trans^(1/3))*(($T_lattice/$T0)^($k_e)))"

term add name=mu_phonon_h eqn = "($B_h/F_trans)+($C_h*(((ND+NA)/$N0)^$lambda_h))/((F_trans^(1/3))*(($T_lattice/$T0)^($k_h)))"

# surface roughness scattering

term add name=mu_surface_e eqn = "(((F_trans^2)/$delta_e+((F_trans^3)/$eta_e))^-1)"

term add name=mu_surface_h eqn = "(((F_trans^2)/$delta_h+((F_trans^3)/$eta_h))^-1)"

term add name=Emob_Lombardi eqn = "(mu_surface_e*mu_phonon_e)/(mu_surface_e+mu_phonon_e)+1"

term add name=Hmob_Lombardi eqn = "(mu_surface_h*mu_phonon_h)/(mu_surface_h+mu_phonon_h)+1"

}

# Dorkel-Leturcq Model

proc mobility.dorkel {Temp} {

set T $Temp

set T0 300

#set D 1.0400e21

#set F 7.4520e13

set	D 2.0e17

set	F 8.28e8

term add name=mob_conwell_dorkel eqn = "($D*(($T)^(3/2))/sqrt(Elec*Hole))*(log(1+($F*($T)^2)*(Elec*Hole)^(-1/3)))^(-1)"

set mul0_e 1430

set A 4.61e17

set B 1.52e15

term add name=mu_L_e eqn = "$mul0_e*($T/$T0)^(-2.2)"

term add name=mu_I_e eqn = "(($A*$Temp^(3/2))/(ND+NA))*(log(1+($B*$T*$T)/(ND+NA))-($B*$T*$T)/((ND+NA)+$B*$T*$T))^(-1)"

term add name=X_e eqn = "sqrt(6*mu_L_e*(mu_I_e+mob_conwell_dorkel)/(mu_I_e*mob_conwell_dorkel))"

term add name=Emob_Dorkel_fix eqn = "mu_L_e*(1.025/(1+(X_e/1.68)^(1.43))-0.025)"

term add name=Emob_Dorkel eqn = "(Emob_Dorkel_fix>0.5)?(Emob_Dorkel_fix):(0.5)"

set mul0_h 495

set A_h 1.0e17

set B_h 6.25e14

term add name=mu_L_h eqn = "$mul0_h*($T/$T0)^(-2.2)"

term add name=mu_I_h eqn = "(($A_h*$Temp^(3/2))/(NA+ND))*(log(1+($B_h*$T*$T)/(NA+ND))-($B_h*$T*$T)/((NA+ND)+$B_h*$T*$T))^(-1)"

term add name=X_h eqn = "sqrt(6*mu_L_h*(mu_I_h+mob_conwell_dorkel)/(mu_I_h*mob_conwell_dorkel))"

term add name=Hmob_Dorkel_fix eqn = "mu_L_h*(1.025/(1+(X_h/1.68)^(1.43))-0.025)"

term add name=Hmob_Dorkel eqn = "(Hmob_Dorkel_fix>0.5)?(Hmob_Dorkel_fix):(0.5)"

}

#Masetti Model

proc mobility.masetti {} {

set mu_max_e 1417.0

set mu_e_min1 52.2

set mu_e_min2 52.2

set mu_e1 43.4

set Pc_e 0.0

set Cr_e 9.68e16

set Cs_e 3.34e20

set a_e 0.68

set b_e 2.0

set mu_max_h 470.5

set mu_h_min1 44.9

set mu_h_min2 0.0

set mu_h1 29.0

set Pc_h 9.23e16

set Cr_h 2.23e17

set Cs_h 6.1e20

set a_h 0.719

set b_h 2.0

term add name=mob1e eqn = "$mu_e_min1*exp(-$Pc_e/(ND+NA))"

term add name=mob2e eqn = "($mu_max_e-$mu_e_min2)/(1+(((ND+NA)/$Cr_e)^($a_e)))"

term add name=mob3e eqn = "$mu_e1/(1+(($Cs_e/(ND+NA))^($b_e)))"

term add name=Emob_Mas eqn = "mob1e+mob2e-mob3e"

term add name=mob1h eqn = "$mu_h_min1*exp(-$Pc_h/(ND+NA))"

term add name=mob2h eqn = "($mu_max_h-$mu_h_min2)/(1+(((ND+NA)/$Cr_h)^($a_h)))"

term add name=mob3h eqn = "$mu_h1/(1+(($Cs_h/(ND+NA))^($b_h)))"

term add name=Hmob_Mas eqn = "mob1h+mob2h-mob3h"

}

#Arora Model

proc mobility.arora {Temp} {

set Amin_e 88

set Amin_h 54.4

set alpham_e -0.57

set alpham_h -0.57

set Ad_e 1252

set Ad_h 407

set alphad_e -2.33

set alphad_h -2.33

set An_e 1.25e17

set An_h 2.35e17

set alphaN_e 2.4

set alphaN_h 2.4

set Aa_e 0.88

set Aa_h 0.88

set alphaa_e -0.146

set alphaa_h -0.146

set T $Temp

set T0 300.0

term add name=mu_min_e eqn = "$Amin_e*($T/$T0)^$alpham_e"

term add name=mu_d_e eqn = "$Ad_e*($T/$T0)^$alphad_e"

term add name=N0_e eqn = "$An_e*($T/$T0)^$alphaN_e"

term add name=Astar_e eqn = "$Aa_e*($T/$T0)^$alphaa_e"

term add name=Emob_Arora eqn = "mu_min_e+(mu_d_e/(1+((ND+NA)/N0_e)^Astar_e))"

term add name=mu_min_h eqn = "$Amin_h*($T/$T0)^$alpham_h"

term add name=mu_d_h eqn = "$Ad_h*($T/$T0)^$alphad_h"

term add name=N0_h eqn = "$An_h*($T/$T0)^$alphaN_h"

term add name=Astar_h eqn = "$Aa_h*($T/$T0)^$alphaa_h"

term add name=Hmob_Arora eqn = "mu_min_h+(mu_d_h/(1+((ND+NA)/N0_h)^Astar_h))"

}

#-------------------------------------------------------

# 2.3 Canali Mobility

#

# C. Canali et al., (1975). “Electron and Hole Drift Velocity

# Measurements in Silicon and Their Empirical Relation to

# Electric Field and Temperature,” IEEE Transactions on

# Electron Devices, vol. ED-22, no. 11, pp. 1045–1047.

# INPUTS Required:

# 1. mob_low - low field mobility

# OUTPUT Terms:

# 1. Emob_Canali - Electron Mobility

# 2. Hmob_Canali - Hole Mobility

#-------------------------------------------------------

proc mobility.canali { $Emob_low $Hmob_low } {

set vsatn 1.07e7

set vsatp 8.37e6

set Emob_Canali "$Emob_low / (sqrt(1 + ($Emob_low*dot(DevPsi,y*1.0e-4) / $vsatn)^2))"

set Hmob_Canali "$Hmob_low / (sqrt(1 + ($Hmob_low*dot(DevPsi,y*1.0e-4) / $vsatp)^2))"

}

#=======================================================

# Section 3. Initial Guess

#=======================================================

#--------------------------

# Initial Guess Commands

# Assumes charge neutrality

#--------------------------

proc InitialGuess {Doping} {

sel z= {(Doping>0.0)

?  ( 0.025*log( (Doping+1.0e10) / 1.0e10))

:  (-0.025*log(-(Doping+1.0e10) / 1.0e10))} name = DevPsi

sel z=1.0e10*exp(DevPsi/0.025) name=Elec

sel z=1.0e10*exp(-DevPsi/0.025) name=Hole

sel z=0.0 name=Qfn

sel z=0.0 name=Qfp

}

proc InitialGuessQF {Doping} {

sel z= {(Doping>0.0)

?  ( 0.025*log( (Doping+1.0e10) / 1.0e10))

:  (-0.025*log(-(Doping+1.0e10) / 1.0e10))} name = DevPsi store

sel z=1.0e10*exp((DevPsi)/0.025) name=Elec store

sel z=1.0e10*exp((-DevPsi)/0.025) name=Hole store

sel z=0.0 name=Qfn store

sel z=0.0 name=Qfp store

}

proc InitialGuessSB {Doping} {

sel z= {(Doping>0.0)

?  ( 0.025*log( (Doping+1.0e10) / 1.0e10))

:  (-0.025*log(-(Doping+1.0e10) / 1.0e10))} name = DevPsi store

sel z=1.0e10*exp((DevPsi)/0.025) name=Elec

sel z=1.0e10*exp((-DevPsi)/0.025) name=Hole

sel z=1.0 name=SBn store

sel z=1.0 name=SBp store

}

#=======================================================

# Section 4. Recombination-Generation

#=======================================================

#--------------------------

# 4.1 Recombination SRH

#

# INPUTS Required:

# 1. ND and NA

# 2. Elec and Hole

# OUTPUT Terms:

# 1. R_SRH

#--------------------------

proc RG.SRH {} {

#SRH Recombination (Simplified)

set tau_max_e 1.0e-5

set tau_max_h 3.0e-6

#set tau_max_e 1.0e-11

#set tau_max_h 3.0e-7

set Nref 1.0e16

#Doping Dependence

term add name=tau_n eqn = "$tau_max_e/(1+(((NA+ND)/($Nref))))"

term add name=tau_p eqn = "$tau_max_h/(1+(((NA+ND)/($Nref))))"

term add name=R_SRH eqn = "((Elec*Hole)-(1.0e20))/(tau_p*(Elec+1.0e10)+tau_n*(Hole+1.0e10))"

}

#--------------------------

# 4.2 Recombination Auger

#

# INPUTS Required:

# 1. Elec and Hole

# OUTPUT Terms:

# 1. R_Auger

#--------------------------

proc RG.Auger {} {

set A_Ae 0.67e-31

set A_Ah 0.72e-31

set B_Ae 2.45e-31

set B_Ah 4.50e-33

set C_Ae 2.2e-32

set C_Ah 2.63e-32

set H_e 3.46667

set H_h 8.25688

set N0 1e18

set T 300.0

term add name=R_Auger eqn = "(CnT*Elec+CpT*Hole)*(Elec*Hole-1.0e20)"

term add name=CnT eqn = "($A_Ae+$B_Ae*($T/$T)-$C_Ae*(($T/$T)^2))*(1+$H_e*exp(-Elec/$N0))"

term add name=CpT eqn = "($A_Ah+$B_Ah*($T/$T)+$C_Ah*(($T/$T)^2))*(1+$H_h*exp(-Hole/$N0))"

}